5. Jetson & Tablet Deployment Report

HTIC (IIT Madras & DBT) Assignment  |  Candidate: Keerthana  |  Model: MobileNetV3-Large (FP16 ONNX) for fetal ultrasound plane classification

# Model & Workload Profile

| Item | Value |
| --- | --- |
| Model | MobileNetV3-Large, FP16 ONNX, 8.05 MB, about 0.44 GFLOPs per image, 224×224 input |
| Measured quality | 92.18% accuracy / 91.16% macro-F1 on 5,271 held-out test images (unseen patients) |
| Measured laptop speed | 4.4 ms inference, 16.6 ms end-to-end (AMD Ryzen 7 5800H CPU, ONNX Runtime) |
| Workload | Classify a single frozen frame on demand; batch-classify archived studies. Live video is a future extension. |
| Key constraints | Low power, quiet / passive cooling, small footprint, moderate cost, patient data kept on-device |

Laptop numbers are measured. All Jetson and tablet latency / power figures below are design-stage estimates and must be validated on the real device.

# Part A: NVIDIA Jetson

## A1. Devices explored and selection

| Jetson device | AI compute | Memory | Bandwidth | Power | Verdict |
| --- | --- | --- | --- | --- | --- |
| Nano (legacy) | 0.47 TFLOPS FP16 | 4 GB | 25.6 GB/s | 5–10 W | Rejected: old software stack, too slow |
| Orin Nano 8 GB | 40–67 TOPS INT8* | 8 GB LPDDR5 | 68–102 GB/s* | 7–25 W* | SELECTED: best fit for size, power and cost |
| Orin NX 16 GB | 100–157 TOPS* | 16 GB | 102 GB/s | 10–25 W+ | Over-specified: extra cost and power unused |
| AGX Orin 64 GB | 275 TOPS | 64 GB | 204.8 GB/s | 15–60 W | Rejected: built for robots and autonomy |

*Upper figures are the "Super" mode (JetPack 6.2). The Orin Nano 8 GB developer kit now lists at $249 (previously $499).

Selection: Jetson Orin Nano 8 GB is the most suitable device: enough compute with a wide margin, small memory footprint, low power, and an affordable price.

## A2. Justification

| Factor | Our model needs | Why Orin Nano fits |
| --- | --- | --- |
| Compute | ~0.44 GFLOPs per image | Tens of TOPS available, so inference uses a tiny fraction and leaves headroom for extra clinical tools |
| Memory | 8.05 MB model | About 0.1% of the 8 GB shared CPU/GPU memory; no memory pressure |
| Latency | Instant result on freeze | TensorRT FP16 expected around 1–2 ms per image (estimate; laptop CPU is 4.4 ms) |
| Power | Quiet, low-heat device | 7–15 W modes; suited to a passive or low-noise enclosure (thermal check needed) |
| Workload | On-demand + batch archives | Handles single frames and batched archive runs on the same board |

## A3. How I would deploy the model

- Convert: export the trained model to ONNX and copy it to the Jetson (JetPack 6.x, in an NVIDIA L4T container).

- Build engine on the device (TensorRT engines are tied to the device and TensorRT version):

trtexec --onnx=mobilenetv3_fetal_fp16.onnx --saveEngine=mobilenetv3_fetal_orin.engine --fp16 --memPoolSize=workspace:1024MiB

- Run: capture frame → resize/normalize (GPU, using VPI or CUDA) → TensorRT FP16 inference → overlay the predicted plane on screen.

- Validate: re-run the 5,271-image test set on the Jetson and accept the engine only if accuracy matches the laptop result (within about 0.1 pp).

## A4. Further optimizations for this hardware

- Move preprocessing to the GPU/VIC: profiling showed loading + preprocessing take about 71% of laptop runtime, so this gives the biggest gain.

- Zero-copy / pinned memory so captured frames reach the GPU without extra copies; CUDA graphs and fixed batch size for stable latency.

- Set the power mode (nvpmodel, jetson_clocks) for consistent timing.

- TensorRT INT8 with proper calibration: worth testing only with an accuracy check, since CPU post-training INT8 failed on this model.

# Part B: AI on Tablets

## B1. Deployment concept

- A handheld ultrasound probe connects to a tablet over USB-C or Wi-Fi. The tablet shows the live scan; when the clinician freezes the image, the app runs the model on the device and shows the detected plane.

- On-device inference means no cloud upload, no network delay, and patient images stay local.

## B2. CPU, GPU, NPU and platform options

|  | Apple iPad (iPadOS) | Flagship Android tablet |
| --- | --- | --- |
| Processor | Apple silicon (A-series / M-series) | Qualcomm Snapdragon 8-series |
| NPU | Apple Neural Engine (ANE) | Qualcomm Hexagon NPU |
| GPU | Apple GPU via Metal | Adreno GPU |
| Main runtime | Core ML (compute units = ALL, so the ANE is used) | ONNX Runtime Mobile with QNN execution provider |
| Alternatives | ONNX Runtime with Core ML provider | LiteRT (TFLite) with GPU delegate, XNNPACK on CPU |
| Precision | FP16 supported natively | NPU generally prefers INT8 (quantized QDQ models) |

| Compute unit | Role for this model | Expected behaviour (estimate) |
| --- | --- | --- |
| CPU | Fallback and preprocessing | About 10–15 ms per image; works everywhere, uses more battery |
| GPU | FP16 inference on Android; backup on iPad | A few ms; higher power than NPU |
| NPU | Primary target | Lowest latency and energy per inference, but depends on operator support and precision |

## B3. Major challenges for this specific model

- INT8 gap: Android NPUs work best with INT8, but my post-training INT8 dropped accuracy by 22–42 pp (depthwise convolutions + Hard-Swish). This needs quantization-aware training, or staying in FP16.

- Operator support: Hard-Swish and squeeze-excite blocks may fall back to CPU on some NPUs, giving partial speedups. Core ML conversion should start from the PyTorch model, since coremltools no longer imports ONNX.

- Preprocessing parity: ultrasound speckle is sensitive to resize method. Different image libraries on each platform can shift accuracy, so every platform must be re-tested on the 5,271-image test set.

- Input domain shift: frames from a probe app (screenshots, different gain and cropping) may differ from the dataset, so accuracy on real device frames still needs validation.

- Battery and heat: not a concern for on-demand single images (very short bursts). It would become one for continuous live video.

## B4. Proposed approach

- iPad (lowest risk): PyTorch → Core ML (FP16) with coremltools, compute units ALL, frames passed as CVPixelBuffer. FP16 is native, so no quantization is needed.

- Android: ONNX Runtime Mobile with the QNN provider on the Hexagon NPU, using a QAT-trained INT8 (or FP16 where the chip supports it). Start with GPU/CPU FP16 as a fallback, then move to the NPU once accuracy is verified. Do preprocessing in native code (NDK).

Reasoning: start with iPad and Core ML because it keeps FP16 accuracy with the least work; add Android once quantization-aware training makes INT8 reliable.

# Summary: Laptop vs Jetson vs Tablet

|  | Laptop (current) | Jetson Orin Nano (proposed) | Tablet (proposed) |
| --- | --- | --- | --- |
| Status | Measured | Design only, estimates | Design only, estimates |
| Runtime | ONNX Runtime CPU, FP16 | TensorRT FP16 | Core ML / ONNX Runtime Mobile |
| Inference latency | 4.4 ms (16.6 ms end-to-end) | ~1–2 ms | ~2–4 ms on NPU |
| Accuracy | 92.18% test | Expected same; verify | Expected same on FP16; verify |
| Cooling | Laptop fans | Heatsink, possibly passive | Passive |
| Best setting | Review desk / PC | Cart or embedded ultrasound unit | Bedside and point-of-care |

Recommendation: Jetson Orin Nano for a cart-based or embedded ultrasound unit, iPad with Core ML for handheld point-of-care use. Both are expected to match laptop accuracy once converted and re-validated.